# IoTStream v1 — Project Phase 2

This folder contains the Phase 2 implementation of the Computer Networks course project “IoT Telemetry Protocol”.

Phase 2 adds the complete protocol, batching, checksum, timestamp masking, impairment testing using Linux netem, reproducible automation, CSV logging, and PCAP capture.

## Project Files

Mini-RFC.pdf

run_experiments.py

client.py

server.py

README.md

## Requirements

Python 3.x

Linux recommended for reproducible impairment tests

Windows supported (tests run manually via Clumsy)

tshark (optional but recommended for PCAP capture)

Root access on Linux for tc qdisc network emulation

## How to Run the Full Experiment Suite

All required baseline + loss + delay/jitter tests are automated.

Linux:

sudo python3 run_experiments.py

This command automatically:

Applies netem settings for each scenario

Starts server.py and client.py

Runs 5 repetitions per scenario (mandatory requirement)

Captures .pcap files using tshark

Logs CSV results

Computes latency, gap counts, duplicate rate, CPU time

Produces summary tables (min/median/max)

## How to Run Client & Server Manually

Start the Server

python3 server.py --host 0.0.0.0 --port 5005 --csv server_log.csv

Server Responsibilities:

Parse 12-byte header

Reorder timestamps (post-analysis)

Detect duplicates

Detect sequence gaps

Log CSV rows:

device_id, seq, timestamp, arrival_time, duplicate_flag, gap_flag, cpu_ms_per_report

Measure CPU cost per packet using process_time()

Start the Client

python3 client.py --host 127.0.0.1 --port 5005 \

--device 1001 --interval 1 --batch 5

Client Behavior:

Sends an INIT packet

For every interval (±10% jitter):

80%: send DATA

20%: send HEARTBEAT

DATA packets include N float readings (N = --batch)

Timestamps masked to 32-bit milliseconds

Header includes checksum over header bytes

## Batching Strategy

Actual Implementation:

Configurable: --batch N

Default: 5 readings per packet

Payload = N floats (4 bytes each)

batch_flag field stores the exact batch size

Why batching?

Reduces per-reading overhead:

Header cost amortized across N readings

Fewer UDP packets → fewer syscalls

Lower CPU time per report

Remains well under the 200-byte payload constraint

Header = 12 bytes

Payload = 4×N bytes

Worst case batch=5 → 12 + 20 = 32 bytes

## Phase 2 Test Scenarios

1. Baseline (1s, 5s, 30s intervals)

No impairment

Expected ≥ 99% packet delivery

Continuous sequence numbers

2. Random Loss (5%)

tc qdisc add dev lo root netem loss 5%

Server must detect sequence gaps

Duplicate rate ≤ 1%

3. Delay + Jitter

tc qdisc add dev lo root netem delay 100ms 10ms

Server logs arrival_time

Packets reordered by timestamp in post-processing

No crash or buffer overflow